CSC3060 Quiz#1  02/05/2026     Student Name:                            Student ID:

Please put your answers in the answer sheets (last two pages).

Section I (30 points): Select the best answer

B

What is computer architecture?

  - It is the hardware implementation of a computer

  - It is the interface between the software and the hardware

  - It is the rules and conventions that a programmer must follow

  - It is the components of a computer, including I/O, CPU, and memory

A

Which of the following is not a system program?

  - ChatGPT

  - Compiler

  - OS

  - Linker

- In the memory, there is a sequence of 128 bits of 0. Which of the following is not a possible representation?

C

It may be two long integer zeros.

  - It may be four single-precision floating-point +0.0

  - It may be a character string “000000000000000”

  - It may be one quad-precision floating-point value

B

What illusion does a memory hierarchy give you?

  - The memory space is all yours.

  - A memory that is fast, large, and inexpensive

  - A memory that is small, large, and expensive

  - A memory that is fast, small, and inexpensive

D

Which of the following phases is not part of the “gcc” command?

  - Preprocess

  - Linking

  - Compiling

  - Loading

B

6) Which of the following architectures has only 16 general-purpose registers

a) RV32MI

b) RV32EM

c) RV64IM

d) RV32ME

C

7) Which of the following descriptions of RV32I ISA is NOT True?

a) It is for 32bit processor implementations; the address range is limited to 4GB

b) Its memory is byte-addressable

c) It requires an on-chip cache hierarchy implemented

d) It supports 32 general-purpose registers: X0-X31

8)  When a program is interrupted, what information must be saved so the program can resume after the interrupt condition is resolved?

C

a) The memory

b) All the files used by this application program

c) The architectural states

d) The cache content

A

9)   How do you set up a 32-bit constant 0xAABBCCDD in register X2?

a) Li, X2, 0xAABBCCDD

b) Addi X2, X0, 0xAABBCCDD

c) LUI, X2, 0xAABBC; ADDi X2, X2, 0xCDD

d) LW X2, 0xAABBCCDD(X0)

10) Why do RISC machines often require data to be naturally aligned?

D

a) Address generation would be easier

b) To make the data memory more compact

c) To support the C and E extension

d) To avoid data items crossing cache lines or page boundaries

D

11) What range can an indirect Jump (JALR) instruction in RV32I reach?

a) 512KB

b) 1MB

c) 4MB

d) 4GB

12) When implementing the add(a, b) function in Project 1, what does the

B

expression (a & b) << 1 represent?

a) The bitwise sum without any carries.

b) The values of the carry bits that need to be added in the next iteration.

c) The absolute difference between the two numbers.

d) A logical shift to check for overflow.

13) In the subtract(a, b) function, how is the subtraction operation typically

C

transformed to use the add() function?

a) By calculating add(a, ~b) directly.

b) By performing a bitwise XOR and then a right shift.

c) By transforming it into add(a, add(~b, 1)) using Two's Complement logic.

d) By using a loop to count the difference between a and b.

14) On a RV64IFD processor, for the following struct

E

Struct  S1 { int white;  long count; char c; long red; int green; char d; int blue} A[1000]; Assume the starting address of A is stored in register X1. What instruction should be used for A[10].blue=0?

a)   SW x0, 322(x1)

b)   SW x0, -520(x1)

c)    SW x0, 340(x1)

d)   SW x0, 4840(x1)

e)   SW x0, 520(x1)

15) Which decimal numbers have exact representations in IEEE 754 FP format?

A

a) 1.625

b) 1.405

c) 1.35

d) 1.245

e) 1.11

Section II (40 points): Multiple Choice (one or more correct answers)

A, C

Which of the following is the representation of 205010 in 16bit.

- 0b 0000100000000010

- 0x 0813

- 0x 0802

- 0b 0000100000010011

- What is ABI?

A,C,D

ABI stands for Application Binary Interface

- ABI is just like API

- ABI includes the OS system calls, the ISA, and the calling convention

- ABI defines whether an application binary is compatible with a computer system.

B,C,D

Which of the following instructions belong to RV64I instead of RV32I?

a)	SRA  X1, X6, X7

b) 	LD X1, 0(X2)

b)	LWU  X1, 0(X2)

c) 	SLLI  X5, X6, 35

A,B,D

4) The contents of three floating-point registers F1, F2, and F3 are as follows:

F1 = 0x40800000

F2= 0x3F800000

F3= 0xC0800000

Which of the following is correct?

a) (F3+F1) == 0.0

b) F3 < F2 < F1

c) F2 = F1/F3

d) F3 = -(F1*F2)

5) Which of the following statements about Jump instructions are true?

A,C,D

Jump instructions (JAL and JALR) have 20-bit and 12-bit immediate.

- Jump instructions are in UJ type

- Jump instructions can be used for function calls

- JALR has a larger jump range than JAL.

6) Which of the following number systems has both +0 and -0?

A,C,D

Signed magnitude integer

- Two’s complement integer

- One’s complement integer

- IEEE-754 Floating-point numbers

7) Which of the following statements about the branch and the store is TRUE?

B,D

a) Both instructions generate a 12-bit  (± 211)  byte offset

b) The immediate in the store instruction is an offset relative to rs1, and is

relative to PC in the branch instruction

c) Both instructions generate a 12-bit offset relative to the base register rs1.

d) Branch instructions lack the “rd” field.

8)  Assume A, B, C are int and X, Y, Z are floating point variables. Which of the following expressions are always TRUE?

D

A2 >= 0

- (X+Y)+Z == X+(Y+Z)

- If (A < 0) then –A > 0

- A == (int)(double)A;

9) Assume variable A is an integer. Under what conditions would the results from a big-endian machine be different from those of a little-endian machine?

A,D

a) Store A as an int but read it as a sequence of char.

b) Store A as a long, and read it as a long

c) Store A as an int, and read it as an int.

d) Store A as a sequence of char, but read it as an int

10) What does 32 mean in RV32I?

B,D

a) The ISA has 32 general-purpose registers

b) The ISA can address up to 2^32 bytes

c) The instructions are 32-bit long

d) The general-purpose registers are 32-bit wide

e) The ISA contains 32 instructions

Section III (50 points): Short and brief answers

- In May 2017, the RISC-V F/D extension added a new R4-type instruction to support the fused-multiply-and-add instruction (FMA), which computes A = B * C + D. However, this is the only R4-type instruction in the RV ISA. (8 points)

- Could you redesign this instruction format using only three operands so that no new R4 type is needed? Your new format should preserve most of the benefits of the FMA instruction. (3 points)

- Undoubtedly, your design in (a) would have broken the regularity of the RV ISA. Please explain what regularity has been disrupted. (2 points)

- Floating-point multiply is typically a 5-cycle operation, and the floating-point add is a 4-cycle operation. Please explain why the FMA is still a 5/6-cycle operation rather than a 9-cycle operation. (3 points)

2)  bfloat16 is a 16-bit numerical floating-point format designed by Google Brain specifically for deep learning. It is now supported by all major high-performance GPU vendors (Google, NVIDIA, Intel, and ARM). (6 points)

a)   IEEE 754-2008 defined the half-precision (FP16) format. Both FP16 and

BFloat16 are widely supported by GPUs. Why do GPUs use such a low-precision floating-point format?  (3 points)

b)  Why is bfloat16 more popular than the IEEE FP16 format? (3 points)

3) Assume your machine is RV32I. For the following function:

Int function_A (unsigned int a; int b,c;)

What RV32I instructions would be best for

a) a*4 		(1 point)

b) b*61 	(2 points)

c) a/8    	(1 point)

d) b >> 3	(1 point)

e) b/8   		(3 points)

f)  a%128     	(2 points)

g)  pow(2,a) 	(2 points)

h)  a=(b >= c)	(2 points)

i)   b*c		(4 points)

Note that a, b, and c are passed in registers according to the calling convention. (2 points)

4) The Intel Itanium architecture includes 128 general-purpose registers. Having more registers would allow the compiler to allocate extra variables to registers, thereby reducing memory-reference instructions. Should we consider incorporating 128 registers in the RV64I ISA? Please elaborate. (6 points)

5) When we access an array, say, A[i], we need a base register for the starting address of A[] and an index register for the value i. So, why isn't there a load instruction with the format LW rd, (rs1+rs2)? (5 points)

6) What are the estimated latencies (in terms of clock cycles) of the following instructions in RV32IF? (5 points)

|  | Estimated latency |
| --- | --- |
| Integer multiply | 3-4 or 3-5 |
| Integer divide | 32 |
| SP FP add | 4 |
| SP FP multiply | 5 |
| FP to Int conversion | 3-5, or 3+ |

| Section 1 | 1 | 2 | 3 | 4 | 5 | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | B | A | C | B | D |  |
|  | 6 | 7 | 8 | 9 | 10 |  |
|  | B | C | C | A | D |  |
|  | 11 | 12 | 13 | 14 | 15 |  |
|  | D | B | C | E | A |  |

| Section 2 | 1 | 2 | 3 | 4 | 5 | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | AC | ACD | BCD | ABD | ACD |  |
|  | 6 | 7 | 8 | 9 | 10 |  |
|  | ACD | BD | D | AD | BD |  |

| Section 3 |  |
| --- | --- |
| 1 | a) Support A = B*C + A instead of A = B*C + D. Therefore, only three operands are needed. One of the source operands, rs3, is the same as rd . This instruction can calculate the dot product, which effectively supports matrix multiplication, the most important use case for FMA. |
|  | b) In RV, the destination register (rd) is the write target, but never a source operand. The above 3-operand instruction broke the regularity. |
|  | c) The 5 cycle fmul operation is based on the use of the parallel adders. The parallel addition requires a reduction step to reduce many partial results into one. This takes a few cycles. The fadd operand can perform exponent alignment simultaneously. When the reduction result is out, the fadd operand has been adjusted, and add to the multiplication result. The multiplication result does not need to be normalized and rounded before adding it to the fadd operand. This saves two cycles, and the fadd exponent alignment saves one cycle. |
| 2 | a) Lower precision format enables higher memory bandwidth and computational bandwidth. For example, a double load can load 4 bfloat16 values into a register. One AVX-512 instruction can compute 32 bfloat16 or fp16 operations per clock cycle. |
|  | b) bfloat16 has an 8-bit exponent, while fp16 has only 5. Bfloat16 can cover a much larger range. The range of bfloat16 is the same as the single-precision FP format, so bfloat16 is more compatible to SP FP than fp16. AI is about predictions. It is an approximate computation; the results do not have to be exact. So bfloat16’s lower precision becomes less critical. |
| 3 | Based on the calling convention, a, b, and c are passed via registers a0, a1, and a2. a) a*4 SLLI t0, a0, 2 b) b*61 SLLI t0, a1, 6; SLLI t1, a1, 2; Sub t0, t0, t1; Add t0, t0, a1; c) a/8 SRL t0, a0, 3 d) b >> 3 SRAI a1, a1, 3 e) b/8 BGE a1, X0, DO-it ADDi a1, a1, 7 DO-it: SRAi a1, a1, 3 f) a%128 ANDI t0, a0, 0x000007F g) pow(2,a) Addi t1, x0, 1; SLL t1, t1, a0 h) a=(b >= c) SLT a0, a2, a1; XORI a0, a0, 1; i) b*c xor a0, a0,a0 # 1. Initialize result = 0 mult_loop: andi t0, a2, 1 # 2. Get LSB of c beqz t0, skip_add # 3. If LSB is 0, skip the addition add a0, a0, a1 # 4. a0 = a0 + b skip_add: slli a1, a1, 1 # 5. b = b << 1 srli a2, a2, 1 # 6. c = c >> 1 bnez a2, mult_loop |

| 4 | NO! Because - Smaller is faster, register access latency will increase - Hardware cost is higher. - Instruction format issue (needs 7b per operand), this may increase instruction size. - Reg saving/restoring overhead is significantly increased at function calls and context switching - Arrays/struct/strings are not allocated in registers, so most memory instructions would not be reduced. |
| --- | --- |
| 5 | When accessing A[i], a base register for the starting address of A[] and an index register for the value i are needed. Assume the base address is in rs1, and the index i is in register rs2. 1) LW rd, (rs1+rs2) does not work. The address calculation should be (rs1 + rs2*element_size). If we add the scaling to rs2 in the load instruction, the addressing mode becomes too complex. If we must use one extra instruction to compute rs2*element_size, this mode does not save on instruction count. 2) This instruction specifies three operands: rs1, rs2, rd. Therefore, it is an R-type instruction. However, R-type instructions are used only for ALU operations. Using this addition would break the regularity of the ISA. 3) In programs, A[i] is often accessed in loops, so when the index i is incremented, the compiler can simply use addi 1 (or 2, 4, 8) to the base address register rs1 without needing to calculate index*element_size. |