CSC3060 Quiz#2  04/02/2026     Student Name:                            Student ID:

Please put your answers on the answer sheets (the last two pages).

Section I (30 points): Select the best answer

- Branch prediction should be performed at which of the pipelined stages?

A

IF

  - ID

  - EXE

  - MEM

  - WB

2) Which branch/jump instruction generated from the following C-statements is most difficult to predict?

C

a)	IF

b)	For

c)	Switch

d)	While

e)	Function call

3) Which of the following microarchitectures may have a CPI lower than 1?

A

a) Superscalar implementation

b) Single-cycle data path implementation

c) Multi-cycle data path implementation

d) Pipelined implementation

4)  In the RISC-V calling convention, which of the following registers is a callee-save register?

D

a)	X0

b)	ra

c)	t1

d)	s1

e)	a1

5)  Which of the following implementations would have the highest instruction execution throughput?

D

a)	Single-cycle data path

b)	Multi-cycle data path

c)	Pipelined data-path

d)	Superscalar and pipelined data path

6)  The ARM Cortex-A57 is a processor with 16 pipeline stages. The decision for a branch instruction is made quite late, near the end of the pipeline. Assume 20% of the instructions executed are branches, and the average CPI is 1. Without branch prediction, what would be the performance penalty caused by branches in A57?

B

a)	90%

b)	75%

c)	50%

d)	20%

7) Why do some RV pipelined data path designs utilize a comparison unit to execute the compare-and-branch instructions instead of using the standard ALU?

B

To avoid structural hazards

- To avoid setting the wrong branch condition due to subtraction overflow

- Because the comparison unit does not trap

- ALU is reserved to compute PC+immediate

8) Which of the following control signals is most difficult to generate?

D

a)	ALUsrc (ALU source)

b)	PCsrc (PC source)

c)	Wren (Write enable)

d)	ALUctr (ALU control)

e)	MemRd (Memory read)

D

9) Which pipelined register stores the least amount of information?

a)	IF/ID

b)	ID/EX

c)	EX/MEM

d)	MEM/WB

10) Which DRAM is used in Nvidia’s high-end GPU global memory?

B

a)	Traditional DRAM

b)	HBM

c)	SDRAM

d)	RDRAM

e) 	DDR5 DRAM

11) How do we index to a branch history table in the correlated branch prediction

A

(also known as the 2-level adaptive branch prediction)?

- The current PC ⊕ the global history shift register value

- The PC address of the current branch instruction

- The current PC + the value of the global history shift register

- The current PC  ⊕  Branch address

12) Which dependency type is the “True Dependence” in code scheduling?

B

a)  WAW (Write After Write)

b) RAW (Read After Write)

c) WAR (Write After Read)

d) RAR (Read After Read)

- In your Project#2, which of the following commands prints only the content of register a0 in decimal?

A

print a0

- info registers

- x/d a0

- print/x a0

C

Regarding Project#2: Bomb Lab, which of the following statements is correct?

- The only way to solve Bomb Lab is by brute-force guessing.

- The bomb is an executable file that can run on any RISC-V platform.

- The goal of Bomb Lab is to determine the correct inputs for each phase without causing the bomb to explode.

- In RV32I, argument registers such as a0 and a1 will stay unchanged throughout the entire function lifetime.

- RISC-V would be a fixed-length ISA if it did not have which extension?

C

A

- B

- C

- D

- F

Section II (40 points): Multiple Choice

- Which of the following codes’ performance is limited by operation latency rather than bandwidth?

A,C,D

A single query checking your account balance

  - Large matrix multiplication

  - Chasing a linked list to locate the end of the list

  - Traversing a binary tree to find a data item

- In which situations should caller-saved registers be considered allocation?

A,C

Leaf routines

- A temporary value used across multiple calls

- Temporaries in expression evaluation

- Array variables

- Local scalar variables

- What hazards might commonly occur when performing a binary tree search on pipelined data path?

C,D

Tree hazards

  - Structure hazards

  - Data hazards

  - Control hazards

- In the RV32I processor data path, what are the possible selections for the address of the next instruction to be fetched?

B,C,D

PC

  - PC + 4

  - PC + immediate

  - RS1+immediate

  - (RS1 + immediate) (i.e., memory indirection)

A,D

Most modern microprocessors have split I/D caches. Why?

a) To avoid structural hazards in the pipeline

b) To get a higher overall cache hit rate

c) To increase processor reliability

d) To increase the cache access bandwidth

A,B,D

Under what circumstances might a longer/deeper pipeline be less desirable?

  - When there are frequent branch mispredictions

  - When there are intensive data dependencies among instructions

  - When there are frequent memory operations

  - When there are lots of instruction cache misses

  - When there are frequent procedure calls

A scheduler gives the following schedule for the dependency graph on the right. Which of them are valid scheduling (i.e., produce the same result as the unscheduled code block)?

- 0 1 2 5 3 9 4 7 6 8 10

A,B,D,E

0 2 1 5 3 8 4 7 9 6 10

- 0 2 4 1 3 7 9 6 5 8 10

- 0 1 2 3 5 4 7 8 6 9 10

- 0 1 2 3 4 5 6 7 8 9 10

8) Which of the following are techniques used to reduce the penalty caused by control hazards?

B,C,D

or B,C

a) Pipelined stalls

b) BTB

c) untagged BHT

d) Always predict backward branches taken

e) Data forwarding

9) What locality is exhibited by the following C loop?

While (A != NULL) A = A->next;

D,E

Data temporal locality

  - Data spatial locality

  - Structural locality

  - Instruction temporal locality

  - Instruction spatial locality

10) What information is stored in each cache block (line)?

B,C,D

a)	Byte offset

b)	Cache block (line) data

c)	Cache block (line) tag

d)	A valid bit

e)	A set number

Section III (50 points): Short and brief answers

- Data Hazards (13 points)

Assume we have the following RV32I code sequence

      - LW 	S1, 8(S0)

      - LW	 S2, 16(S0)

      - SUB	T4, S2, S1

      - AND	T5, T4, T3

      - SUB	S4, T4, T3

      - ADD	T6, S4, T4

      - SW	T6, 4(S7)

      - LW	S3, 24(S7)

      - LW        S1, 8(S7)

      - LW	S5, 32(S0)

      - ADD	T7, S3, S1

      - LW	S6, 64(S0)

- In a 5-stage pipelined data path, many data hazards will be detected. How many bubbles will be generated during the execution without data forwarding? (3)

9 bubbles  2-3: 2 bubbles, 3-4: 2, 5-6: 2, 6-7: 2, 9-11: 1

- Now, if we add data forwarding, how do you detect the data forwarding conditions in the pipeline? For example, writing down the forwarding signal generation equations (e.g., PipelineReg1.xx == PipelineReg2.yy). Please list all possible data-forwarding conditions. (4)

To forward data from the MEM/WB pipelined register

a) MEM/WB.RegRd = ID/EX.RegRs1 or

MEM/WB.RegRd = ID/EX.RegRs2

Instruction 2-3, 9-11

To forward data from the EX/MEM pipelined register

b) EX/MEM.RegRd = ID/EX.RegRs1 or

EX/MEM.RegRd = ID/EX.RegRs2

Instruction 3-4,5-6,6-7

- With data forwarding, how many bubbles remain? (2)

1 bubble remains

- Without data forwarding, we may still avoid bubbles by reordering the instructions. Please provide one reordered code sequence that is free of data hazards. (4)

1.	LW 	S1, 8(S0)

2.	LW	 S2, 16(S0)

8.	LW	S3, 24(S7)

9.	LW        S8, 8(S7)

3.	SUB	T4, S2, S1

10.	LW	S5, 32(S0)

11.	ADD	T7, S3, S8

4.	AND	T5, T4, T3

5.	SUB	S4, T4, T3

12.	LW	S6, 64(S0)

6.	ADD	T6, S4, T4

7.	SW	T6, 4(S7)

Hazards have been reduced, but this schedule is not free of data hazards.

Anyone who attempts to come up with a legal schedule with reduced hazards gains 2 points

Any schedule that reduces 4 or more bubbles earns 3 points.

Anyone mentioned that instructions 10 and 12 may not be scheduled before instruction 7 receives full credit (4 points).

2) Register ports (4 points)

    - How many write ports and read ports must we provide for the RV32I register file? (2)

1W 2R

    - The FMA (Floating point fused multiply-and-add) instruction in R4 format was officially ratified in the F extension in 2019. Suppose we need to implement a two-issue superscalar RV64IFD processor with FMA instructions; how many read and write ports must the floating-point register file provide? (2)

6R 2W

3)Memory Wall (8 points)

- What is “memory wall”? (2)

Memory wall refers to the growing gap between the speed of processors and the speed of main memory

- The memory wall can be divided into the latency and the bandwidth walls. What could be done to mitigate the impact of each? (3)

Latency wall:  Latency reduction via Cache or Local memory. Latency hiding via multi-threading

Bandwidth wall: HBM, multiple banks, wider bus, …

- How is the memory hierarchy managed? By hardware, software, or by a combination? (3 points)

Register and local memory: managed by software

Cache: managed by hardware

Main memory: managed by a combination of hardware and OS.

- Architecture/Micro-architecture states (10 points)

In a processor, we have lots of storage elements, for example

- General-purpose registers

- PC

- Vector registers

- Floating-point registers

- Overflow/Underflow Flags

- Pipelined registers

- BHT (Branch History Table)

- BTB (Branch Target Buffer)

- L1 Instruction cache

- L1 Data cache

- Which of the above are architectural states for RV32I? (1)

1 and  2

- Which are architectural states for RV64IFV? (2)

1, 2, 3 and  4

- Why must instructions be retired in order? For example, even if an integer add instruction is issued later, but completes sooner than a divide instruction, it must wait for the divide to finish and update the register. (3)

Architecture states must be updated in-order so that we may obtain a precise architecture state for program resumption.

- Which of the above are microarchitectural states? (2)

6, 7, 8, 9 and 10

- Why, during an interrupt, are micro-architecture states not saved? (2)

Micro-architecture states can be regenerated once a program resumes

- Data Structures and Data Hazards (5 points)

Many basic data structures naturally involve memory dependencies that can slow down algorithm performance on pipelined processors due to data hazards. These include linked lists, trees, queues, stacks, and hash tables with collision chains. As a programmer, what steps can you take to reduce inefficiencies in applications that use such data structures?

- Prefer contiguous data structures

Using Arrays

Optional: Using B/B+ trees instead of a binary tree

Optional: Using open addressing hashing rather than a collision chain

  - Avoid doing one data item at a time

Traverse multiple independent streams together

e.g., batch lookups or updates

Optional: Allow multiple memory requests in flight

6) Control hazards (10 points)

In the pipelined data path, the branch target address (PC + immediate) is calculated in the EXE stage. However, this (PC+immediate) calculation could occur earlier at the ID stage, since both the PC value and immediate are available there.

- If PC+immediate is calculated at the ID stage, how can we take advantage of it? (2)

This would improve predicted-taken branch handling

because it allows IF to fetch the right target earlier.

- Do you think this is a good design choice? You must argue for it or against it. (3)

Most current processors have BTB + BHT to predict both “whether” and “where” for branches. Having PC+immediate computed one stage earlier may not improve performance. This design will add one more adder in the ID stage.

- Modern microprocessors, like APPLE’s M4, can predict branch accuracy up to 99%. Please discuss potential directions for further improvements in branch prediction. (3)

Further improvement in branch prediction would focus on indirect branches: return branch, branches for switch and virtual functions.

d) On the reference textbook, Figure 4.59 suggests that a branch instruction would cause three pipelined bubbles. In class, we claim that a branch instruction may cause only two bubbles. Please explain why there is one bubble discrepancy. (2)

The reference book assumes the branch outcome: the taken/not-taken decision and the branch targets, are available at the end of the EXE stage, but are used to resteer the instruction fetch at the MEM stage. This is why it has 3 bubbles. Without branch prediction, most microprocessors would resteer instruction fetch at the EXE stage when the branch condition and the target address are available.